🌡️  BEGINNER FIRMWARE GUIDE

Aether-Flow

ESP32 Firmware — Step-by-Step Guide

No electronics experience needed. Every concept explained from scratch.

Version 1.0  ·  2025

🧠  WHAT IS FIRMWARE?

Firmware is the software that runs directly on a microcontroller chip — in our case, the ESP32. Unlike apps on your phone, firmware runs with no operating system, no screen, no mouse. It's just pure code talking directly to hardware.

Think of it like this: if the ESP32 is the brain of Aether-Flow, the firmware is the instructions that tell that brain what to do. It wakes up when power is applied, and runs the same loop forever until power is removed.

The Big Picture

The ESP32 firmware does 4 things in a loop: READ sensors → THINK (PID control) → ACT (set cooling power) → REPORT data to the Raspberry Pi AI. The whole loop repeats every 500 milliseconds — twice per second.

🛒  WHAT YOU NEED TO BUY

Component

Cost ~

Where to Buy

ESP32-WROOM-32 dev board

₹400–600

Robu.in, Amazon India, AliExpress

TMP117 temperature sensor

₹350

Robu.in / Adafruit

SHT40 humidity sensor

₹420

Robu.in / Adafruit

INA226 power monitor module

₹280

Amazon India / AliExpress

TEC1-12706 Peltier module

₹120

Amazon India

IRF540N MOSFET (for PWM)

₹30 each

Local electronics store

USB-A to Micro-USB cable

₹80

Any shop

Jumper wires (male-male)

₹100

Robu.in / Amazon

Breadboard

₹80

Any electronics shop

💻  STEP 1 — INSTALL ARDUINO IDE

Arduino IDE is the free software you use to write code and upload it to the ESP32. Think of it like Microsoft Word, but for microcontroller code.

•	Go to arduino.cc/en/software and download Arduino IDE 2.x for your operating system

•	Install it with default settings

•	Open Arduino IDE — you'll see a blank code editor

•	Now add ESP32 support: go to File → Preferences

•	In 'Additional boards manager URLs' paste this link:

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

•	Click OK, then go to Tools → Board → Boards Manager

•	Search 'esp32', find 'esp32 by Espressif Systems', click Install (takes 2-3 minutes)

•	After install: Tools → Board → esp32 → 'ESP32 Dev Module'

✅ How to know it worked

After selecting 'ESP32 Dev Module', the bottom bar of Arduino IDE should show the board name. If you connect your ESP32 via USB, you should also be able to select a COM port under Tools → Port.

📦  STEP 2 — INSTALL LIBRARIES

Libraries are pre-written code packages that handle the complicated parts of talking to sensors. Instead of writing hundreds of lines to communicate with the TMP117, we install a library and use it in 2 lines.

Go to Tools → Manage Libraries in Arduino IDE. Search for and install each of these:

Library Name

Author

What It Does

Adafruit TMP117

Adafruit

Reads the temperature sensor

Adafruit SHT4x

Adafruit

Reads humidity & ambient temp

INA226_WE

Wolfgang Ewald

Reads power consumption

ArduinoJson

Benoit Blanchon

Formats data as JSON for Pi

🔌  STEP 3 — WIRING THE HARDWARE

Before uploading any code, you need to physically connect the components. Use jumper wires on a breadboard. No soldering needed for prototyping.

⚠️ Safety First

Always connect/disconnect wires with the ESP32 UNPLUGGED from USB. Never connect 12V directly to any ESP32 pin — the ESP32 runs on 3.3V logic. The Peltier module connects to 12V through the MOSFET circuit only.

Sensor Wiring (I2C Bus)

All three sensors (TMP117, SHT40, INA226) share the same two wires — this is called the I2C bus. They each have a unique address so the ESP32 knows who it's talking to.

Component

ESP32 Pin

Notes

PWM Output Wiring (Peltier & Fans)

The ESP32 cannot power the Peltier directly — it only outputs 3.3V at tiny current. We use a MOSFET (IRF540N) as a switch: the ESP32 tells the MOSFET to open/close thousands of times per second, controlling how much 12V power reaches the Peltier.

🚀  STEP 4 — UPLOAD THE FIRMWARE

•	Open the file AetherFlow_Firmware.ino in Arduino IDE (File → Open)

•	Connect your ESP32 to your computer with a USB cable

•	In Arduino IDE: Tools → Port → select the COM port that appeared (e.g. COM3 on Windows, /dev/ttyUSB0 on Linux)

•	Tools → Board → ESP32 Dev Module (if not already set)

•	Click the Upload button (right arrow ▶ icon) in the top left

•	Wait for 'Done uploading' message at the bottom — takes 30–60 seconds

•	If you see errors, check that all 4 libraries are installed correctly

🔍 Watch the Serial Monitor

After uploading, go to Tools → Serial Monitor. Set baud rate to 115200. You should see 'AETHER-FLOW FIRMWARE v1.0 STARTING...' and then live sensor readings every 500ms. This is your window into the ESP32's brain.

📖  UNDERSTANDING THE CODE

The Main Loop — What Happens Every 500ms

Every half second, the ESP32 runs through these steps in order:

1. readSensors()      → Collects temperature, humidity, power data

2. checkSafety()      → If anything is dangerous, stops everything

3. readAICommand()    → Checks if Raspberry Pi sent new instructions

4. runPIDController() → Calculates how much cooling is needed (fallback)

5. Apply PWM outputs  → Sends signals to Peltier and fans

6. sendTelemetry()    → Reports all data as JSON to Raspberry Pi

What is PWM?

PWM (Pulse Width Modulation) is how we control power to the Peltier and fans. Instead of a simple on/off switch, PWM switches the power on and off thousands of times per second. By changing how long it stays on vs. off, we control the average power.

PWM value = 0    → Always OFF  (0% power)  → No cooling

PWM value = 127  → 50% ON/OFF  (50% power) → Half cooling

PWM value = 255  → Always ON   (100% power) → Maximum cooling

Our PWM runs at 25,000 Hz — 25,000 cycles per second.

This is above human hearing range, so no annoying whine.

The PID Controller — Your Cooling Brain

The PID controller is the heart of the firmware. It's a mathematical formula that automatically calculates the right amount of cooling to keep temperature at exactly 45°C.

Target temperature:   45.0°C  (TEMP_TARGET)

Current temperature:  52.3°C  (read from TMP117)

Error:                +7.3°C  (too hot!)

P = 2.5 × 7.3  = 18.25  (main correction)

I = 0.08 × acc = small  (fixes lingering error)

D = 1.2 × rate = varies (prevents overshoot)

PID output = P + I + D → converted to PWM duty cycle

The JSON Data Format

Every 500ms the ESP32 sends this line over USB serial to the Raspberry Pi AI:

{"t":47.3,"ta":24.1,"h":52.1,"dp":14.2,"w":12.4,

"v":12.1,"p":120,"f1":180,"f2":180,

"r1":1200,"r2":1180,"mode":"PID","safe":1}

t  = target temperature (°C)

ta = ambient (room) temperature (°C)

h  = humidity (%)

dp = dew point (°C) — condensation danger threshold

w  = power draw (Watts)

p  = Peltier PWM duty (0-255)

f1/f2 = fan duty cycles

r1/r2 = fan RPM (actual speed)

mode = 'AI' or 'PID' (who is in control)

safe = 1 (healthy) or 0 (emergency stop triggered)

🧪  STEP 5 — TESTING YOUR SETUP

Before connecting 12V power, test with just USB and sensors. Here's what to expect:

What You See

What It Means

LED flashes 5 times fast

✅ Firmware loaded and all sensors found

LED blinks slowly (every 500ms)

✅ Main loop is running normally

JSON lines in Serial Monitor

✅ Data is being collected and sent

mode: 'PID' in JSON

✅ Normal — AI not connected yet

[ERROR] TMP117 not found

❌ Check wiring on SDA/SCL pins

LED rapid blinking (non-stop)

❌ Emergency stop triggered — check temp/power

🎯 First Milestone

If you can see live JSON temperature readings in the Serial Monitor, your ESP32 firmware is working correctly! The Peltier and fans won't do anything yet (they need 12V), but the sensing and reporting pipeline is complete.

🚀  WHAT COMES NEXT

Once your firmware is running and you can see sensor data in the Serial Monitor, here are your next steps in order:

•	Connect the MOSFET circuit and 12V supply — test Peltier manually by sending PWM values

•	Connect the fans and verify RPM readings in Serial Monitor

•	Set up the Raspberry Pi and connect it to ESP32 via USB

•	Run the Python AI data collector script on the Pi (AetherFlow_DataLogger.py)

•	Collect 2-4 weeks of temperature/load data for AI training

•	Train the LSTM prediction model on that data

•	Deploy the trained model to the Pi — it will now send JSON commands to the ESP32

•	Watch the mode switch from 'PID' to 'AI' in your Serial Monitor — that's the moment Aether-Flow becomes intelligent

You're building the future of cooling.

Aether-Flow — Where molecular precision meets machine intelligence.